VOLUME 4.3

FINANCIAL OFFER TEMPLATES

UNIT PRICE CONTRACTS

Content

4.3.1 — Introduction

4.3.2 — Bill of quantities

4.3.3 — Price schedule

4.3.4 — Dailyworks schedule

4.3.5 — Detailed breakdown of prices

VOLUME 4.3.1 — INTRODUCTION

1. 	General

1.1	The bill of quantities (Volume 4.3.2) is the document containing an itemised breakdown of the works to be carried out in a unit price contract, indicating a quantity for each item and the corresponding unit price. The quantities set out in the bill of quantities are estimated quantities. Each price for each item of the bill of quantities is detailed in the price schedule (Volume 4.3.3).

The amounts due will be calculated by measuring the actual quantities of the works executed and by applying the unit rates to the quantities actually executed for each item.

1.2	The detailed breakdown of prices (Volume 4.3.5) is the list containing the basic costs, net costs and mark-ups from which each price on the bill of quantities and the price schedule and on the daywork schedule results.

The detailed breakdown of prices provides the coefficients for applying the price revision formula referred to in Article 48.2 of the special conditions and can provide the basis for valuation of additional work ordered referred to in Article 37.5 of the general conditions.

1.3	Provisional sums for use when works are to be executed on a dailywork basis (Volume 4.3.4) can only be executed by administrative order of the supervisor in accordance with the terms of the contract.

2.	Specific to Volumes 4.3.2, 4.3.3 and 4.3.4

2.1	The prices inserted in the bill of quantities and price schedule are to be the full inclusive values of the works described under the items, including all costs and expenses that may be required in and for the construction of the works described, together with any temporary works and installations which may be necessary and all general risks, liabilities and obligations specified or implied in the documents on which the tender is based. It will be assumed that establishment charges, profit and allowances for all obligations are spread evenly over all unit rates.

2.2	Save where the technical specifications or the bill of quantities and the price schedule specifically and expressly state otherwise, only permanent works are to be measured.

2.3	No allowance will be made for loss of materials or volume thereof during transport or compaction.

2.4	The prices of the bill of quantities, price schedule and dailywork scheduleare all-inclusive and include any non-exonerated tax or fiscal duty.

2.5	The unit prices in Volumes 4.3.2 and 4.3.4 are obtained by multiplying the net cost of Volume 4.3.5 - Table D, on the one hand, with the coefficient K, on the other hand.

The coefficient K represents the proportions between the Site costs (Fc) of Volume 4.3.5 - Table E and the Total net costs (Ps) and between the General costs (Fg) of Volume 4.3.5 - Table F and the Contract price (Pv): K = (1+A)/(1-B) whereby

A= Fc/Ps

B =Fg/Pv

2.5	The units of measurement used in the annexed technical documentation are those of the International System of Units (SI). No other units may be used for measurements, pricing, detail drawings etc. (Any units not mentioned in the technical documentation must also be expressed in terms of the SI.) Abbreviations used in the bill of quantities are to be interpreted as follows:

mm	means	millimetre

m	means	metre

mm²	means	square millimetre

m²	means	square metre

m³	means	cubic metre

kg	means	kilogram

to	means	tonne (1000 kg)

pcs	means	pieces

h	means	hour

L.s.	means	Lump sum

km	means	kilometre

l	means	litre

%	means	per cent

N.d	means	nominal diameter

m/m	means 	man-month

m/d	means 	man-day

VOLUME 4.3.2 — BILL OF QUANTITIES

- Phase - ARCHITECTURE

| PRICE No | TITLE | UNIT | UNIT PRICE | ESTIMATED QUANTITIES | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| (a) | (b) | (c) | (d) | (e) | (f = d * e) |
| 000-99 | PREPARATION WORKS |  |  |  |  |
| 001 | Planning and clearing the terrain with marking and staking of green islands, flowerbeds, etc., according to elevation data from the location urbanistic part with a surveyor and geometric parameters. | m2 |  | 1.750,00 |  |
| 002 | Mechanical excavation of soil for foundation strips, in a narrow scale, of II-III category with included loading and transportation surplus soil from excavations to a landfill in distance of 20 km. | m3 |  | 20,32 |  |
|  | Concreting of AB retaining wall under the stand of island 1 and island 2 with dimensions 20/50 cm with foundation strip 40/40 cm with MB30 in adequate thickness |  |  |  |  |
| 003 | Stand 1 at Island 1 | m3 |  | 7,94 |  |
| 004 | Stand 2 at Island 2 | m3 |  | 8,44 |  |
|  | Procurement, transportation, fabrication and installation of reinforcement in foundation strips and retaining walls at basketball court bleachers. |  |  |  |  |
| 005 | Stand 1 at Island 1 | kgr |  | 284,24 |  |
| 006 | Stand 2 at Island 2 | kgr |  | 231,93 |  |
|  | PREPARATORY WORKS – FENCING TO NEIGHBOR |  |  |  |  |
| 007 | Mechanical excavation of soil for foundation strips, in a limited volume, of II-III category, including loading, transportation, and unloading of surplus soil from the excavations to a landfill in distance of 20 km | m3 |  | 24,19 |  |
| 008 | Pouring reinforced concrete (AB) foundation strips for a fence to the neighbor with dimensions of 60/60 cm and an RC wall of 20/170 cm using MB 30 concrete as required. | m3 |  | 7,20 |  |
| 009 | Filling and compaction and tamping with varying thickness, in layers of 20-30 cm each, where the minimum compaction modulus of the final cushion layer should be Mv > 40 MPa, and the minimum compaction γ = 21.00 kN/m³, at the foundation strip | m3 |  | 48,73 |  |
| 010 | Procurement, transportation, fabrication, and installation of reinforcement in AB foundation strips and AB retaining wall adjoining the neighbor | kgr |  | 330,23 |  |
| Total 1 | Total of PREPARATION WORKS |  |  |  |  |
|  | GREEN ISLANDS |  |  |  |  |
| 100-200 | SPACE BETWEEN GREEN ISLANDS |  |  |  |  |
| 101 | Mechanical excavation of soil in a large volume, categories 3 and 4, 90% mechanical and 10% manual, for leveling the space between islands, including the removal of excavated soil to a landfill within 20 km. | m3 |  | 213,00 |  |
| 102 | Filling and compaction and tamping with varying thickness, in layers of 20-30 cm each, where the minimum compaction modulus of the final cushion layer should be Mv > 40 MPa, and the minimum compaction γ = 21.00 kN/m³, at the space between the islands | m3 |  | 213,00 |  |
| 103 | Procurement, transportation, and montage of geotextile (felt) placed on a gravel substrate to form pathways for movement. | m2 |  | 710,00 |  |
| 104 | Filling and leveling a layer of humus with d=10 cm, in the space between the islands under the pavel elements. | m2 |  | 710,00 |  |
| 105 | Procurement, transportation and installation of square paver elements 60x40 cm, d=10 cm on a substrate of sand and humus for a path. The sand pad is part of this position. | m2 |  | 710,00 |  |
|  | GREEN ISLANDS 1,2,3,4,5,6 |  |  |  |  |
| 106 | Earth filling, mechanical, with leveling and compaction of the terrain to the appropriate density for the entire designated area to create green islands according to the provided details. | m2 |  | 2.042,00 |  |
| 107 | Procurement, transport and installation of agrofolia under the incorporated humus in all green islands. | m2 |  | 1.182,50 |  |
|  | Greening of the areas within the green islands, which include the following positions. |  |  |  |  |
| 108 | topsoil filling + including soil delivery and spreading. | m3 |  | 18.40 |  |
| 109 | greening - grass seeding (leveling the surface, forming a lawn with grass seed) | m2 |  | 92.00 |  |
| 110 | topsoil filling + including soil delivery and spreading | m3 |  | 90.00 |  |
| 111 | greening - grass seeding (leveling the surface, forming a lawn with grass seed | m2 |  | 450.00 |  |
| 112 | topsoil filling + including soil delivery and spreading | m3 |  | 49.00 |  |
| 113 | greening - grass seeding (leveling the surface, forming a lawn with grass seed) | m2 |  | 245.00 |  |
| 114 | topsoil filling + including soil delivery and spreading | m3 |  | 35.40 |  |
| 115 | greening - grass seeding (leveling the surface, forming a lawn with grass seed) | m2 |  | 177.00 |  |
| 116 | topsoil filling + including soil delivery and spreading | m3 |  | 27.60 |  |
| 117 | greening - grass seeding (leveling the surface, forming a lawn with grass seed) | m2 |  | 138.00 |  |
| 118 | topsoil filling + including soil delivery and spreading | m3 |  | 2.60 |  |
|  | transportation, and installation of stone slabs with dimensions of 200/70/12 cm on a concrete base with a thickness of 8 cm to form bleachers at the basketball court |  |  |  |  |
| 119 | island 1 - stone slabs | m2 |  | 27.00 |  |
| 120 | island 1 - lean concrete | m2 |  | 30.00 |  |
| 121 | island 2 - lean concrete | m2 |  | 30.00 |  |
|  | Procurement, transport and planting of a dendrological type of plant defined by the designer |  |  |  |  |
| 122 | mark D - Pinus nigra – (h=1.80m2.00m) | pieces |  | 1 |  |
| 123 | mark E (Cotoneaster dammeri - bella) (Ф=40-50сm) | pieces |  | 2 |  |
| 124 | mark S - (Acer palmatum atropurpureum) (h=1.50-1.60m) | pieces |  | 7 |  |
| 125 | mark E (Cotoneaster dammeri - bella) (Ф=40-50сm) | pieces |  | 2 |  |
| 126 | mark B (Salix babylonica) h=3.0m | pieces |  | 2 |  |
| 127 | mark V - (Alnus spp.) h=2.00m | pieces |  | 1 |  |
| 128 | mark G - (Prunus laurocerasus otto luyken) (Ф=50-60см) | pieces |  | 6 |  |
| 129 | mark A - (Platanus SPP - Platanus × acerifolia) h=4.0м | pieces |  | 6 |  |
| 130 | mark Z - (HEDERA HELIX) (h=50-60cm) | pieces |  | 6 |  |
| 131 | mark E - (Cotoneaster dammeri - bella) (Ф=40-50сm) | pieces |  | 1 |  |
| 132 | mark I - (picea pungens glauca) (h=1.5-1.6m) | pieces |  | 1 |  |
| 133 | mark J1 - (Fagus atropurpurea) (h=1.5-1.6m) | pieces |  | 2 |  |
| 134 | mark J - (Fagus moesiaca) (h=1.5-1.6m) | pieces |  | 1 |  |
| 135 | mark G - (Prunus laurocerasus otto luyken) (Ф=50-60см) | pieces |  | 1 |  |
| 136 | mark Z - Vinca minor) (расад Р9) | pieces |  | 1000 |  |
| 137 | mark B - (Salix babylonica) (h=3.0m) | pieces |  | 1 |  |
|  | Procurement, transportation, and installation of concrete curbstones 7/20/50 cm made of MB 40 placed on a concrete base of MB 20 (park curb) (The concrete base is part of this position) |  |  |  |  |
| 138 | island 1 | m1 |  | 42.00 |  |
| 139 | island 2 | m1 |  | 107.00 |  |
| 140 | island 3 | m1 |  | 114.00 |  |
| 141 | island 4 | m1 |  | 109.00 |  |
| 142 | island 5 | m1 |  | 67.20 |  |
| 143 | island 6 | m1 |  | 13.00 |  |
|  | Procurement, transportation, and planting of a flowerbed with seasonal flowering plants at a rate of 25 flowers per square meter |  |  |  |  |
| 144 | island 3 | pieces |  | 550.00 |  |
| 145 | island 4 | pieces |  | 725.00 |  |
| 146 | island 5 | pieces |  | 175.00 |  |
| Total 2 | Total of GREEN ISLANDS |  |  |  |  |
| 200-300 | URBAN EQUIPMENT |  |  |  |  |
| 201 | Procurement, transportation, and installation of waste bins made from wooden planks arranged in a circle, held by two rings, made of metal flat bars 40/4 mm (bottom and top) on a metal box-profile frame. The planks should be fixed to the metal flat bars. | pieces |  | 6.00 |  |
| 202 | Procurement, transport and installation of taps for drinking water, according to the investor's choice. The taps should have a built-in valve for turning on and off the water. | pieces |  | 3.00 |  |
| 203 | Procurement, transportation, and installation of benches (the benches to be anchored on two reinforced concrete foundation blocks with dimensions of 20x45 cm and a depth of 25 cm). The seating benches to be made from stamped metal construction of box profiles. | pieces |  | 20.00 |  |
| Total 3 | Total of URBAN EQUIPMENT |  |  |  |  |
| 300-400 | CANOPIES |  |  |  |  |
|  | Preparing, transport and montage of steel canopies miniaturized and oil-painted, consisting of circular profiles F100/4mm and covered with solar panels 100/200 mm placed on steel profiles 120.100.5 mm and substructure made of steel profiles 50.50.4 mm according to all standards for this type of work. It is possible to eventually add couplings depending on the weight of the vertical couplings. |  |  |  |  |
|  | Canopy 1 |  |  |  |  |
| 301 | Tiles 200.200.10 mm | kgr |  | 25.20 |  |
| 302 | Pillars101,6 5 мм / 11,9 kg/m | kgr |  | 309.40 |  |
| 303 | Beams 120.100.5 мм / 22,4 kg/m | kgr |  | 907.20 |  |
| 304 | substructure 50.50.4 мм / 5,51 kg/m | kgr |  | 539.98 |  |
| 305 | substructure 50.50.4 мм / 5,51 kg/m | kgr |  | 293.68 |  |
| 306 | couplings – horizontal | kgr |  | 88.16 |  |
| 307 | sand + lime 10% | kgr |  | 216.36 |  |
|  | Canopy 2 |  |  |  |  |
| 308 | Tiles 200.200.10 mm | kgr |  | 25.20 |  |
| 309 | Pillars101,6 5 мм / 11,9 kg/m | kgr |  | 309.40 |  |
| 310 | Beams 120.100.5 мм / 22,4 kg/m | kgr |  | 907.20 |  |
| 311 | substructure 50.50.4 мм / 5,51 kg/m | kgr |  | 539.98 |  |
| 312 | substructure 50.50.4 мм / 5,51 kg/m | kgr |  | 293.68 |  |
| 313 | couplings – horizontal | kgr |  | 88.16 |  |
| 314 | sand + lime 10% | kgr |  | 216.36 |  |
|  | Canopy 3 |  |  |  |  |
| 315 | Tiles 200.200.10 mm | kgr |  | 25.20 |  |
| 316 | Pillars101,6 5 мм / 11,9 kg/m | kgr |  | 309.40 |  |
| 317 | Beams 120.100.5 мм / 22,4 kg/m | kgr |  | 907.20 |  |
| 318 | substructure 50.50.4 мм / 5,51 kg/m | kgr |  | 539.98 |  |
| 319 | substructure 50.50.4 мм / 5,51 kg/m | kgr |  | 293.68 |  |
| 320 | couplings – horizontal | kgr |  | 88.16 |  |
| 321 | sand + lime 10% | kgr |  | 216.36 |  |
|  | Canopy4 |  |  |  |  |
| 322 | Tiles 200.200.10 mm | kgr |  | 25.20 |  |
| 323 | Pillars101,6 5 мм / 11,9 kg/m | kgr |  | 309.40 |  |
| 324 | Beams 120.100.5 мм / 22,4 kg/m | kgr |  | 907.20 |  |
| 325 | substructure 50.50.4 мм / 5,51 kg/m | kgr |  | 539.98 |  |
| 326 | substructure 50.50.4 мм / 5,51 kg/m | kgr |  | 293.68 |  |
| 327 | couplings – horizontal | kgr |  | 88.16 |  |
| 328 | sand + lime 10% | kgr |  | 216.36 |  |
| 329 | Preparation, transport and montage of horizontal hanging gutters in canopies with R.W=75 cm from plasticized, galvanized steel sheet with d=0.55 mm. The compositions made according to technical regulations, welded and with simultaneous production of fall %. Complete with binding material and necessary brackets (hooks and claws protected against corrosion) | m2 |  | 42.00 |  |
| 330 | Preparation, transport and montage of horizontal hanging gutters in canopies with R.W=75 cm from plasticized, galvanized steel sheet with d=0.55 mm. The compositions made according to technical regulations, welded and with simultaneous production of fall %. Complete with binding material and necessary brackets (hooks and claws protected against corrosion) | m2 |  | 8.40 |  |
| 331 | Mechanical excavation of soil for foundation strips, in a narrow scale, of II-III category with included loading and transportation surplus soil from excavations to a landfill in distance of 20 km. | m3 |  | 69.00 |  |
|  | Concreting of AB single foundations for steel canopy with solar panels of dimensions 80/80/50 cm with pillars 30/30/40 cm with MB 30 in required thickness. |  |  |  |  |
| 332 | Canopy 1 | m3 |  | 2.85 |  |
|  | 80/80/50 см |  |  |  |  |
|  | 30/30/40 см |  |  |  |  |
| 333 | Canopy 2 | m3 |  | 2.85 |  |
|  | 80/80/50 см |  |  |  |  |
|  | 30/30/40 см |  |  |  |  |
| 334 | Canopy 3 | m3 |  | 2.85 |  |
|  | 80/80/50 см |  |  |  |  |
|  | 30/30/40 см |  |  |  |  |
| 335 | Canopy 4 | m3 |  | 2.85 |  |
|  | 80/80/50 см |  |  |  |  |
|  | 30/30/40 см |  |  |  |  |
|  | Procurement, transportation, fabrication and installation of reinforcement inABfoundation singles for canopy. |  |  |  |  |
| 336 | Canopy 1 | kgr |  | 335.68 |  |
| 337 | Canopy 2 | kgr |  | 335.68 |  |
| 338 | Canopy 3 | kgr |  | 335.68 |  |
| 339 | Canopy 4 | kgr |  | 335.68 |  |
|  | Filling and compacting a buffer with a thickness of d=40cm, in layers of 20cm each, where the minimum compaction modulus of the final buffer layer should be Mv>40MPa and the minimum compaction ϒ=21.00kN/m³, under the foundations of steel canopies . |  |  |  |  |
| 340 | Canopy 1 | m3 |  | 32.00 |  |
| 341 | Canopy 2 | m3 |  | 32.00 |  |
| 342 | Canopy 3 | m3 |  | 32.00 |  |
| 343 | Canopy 4 | m3 |  | 32.00 |  |
|  | Procurement, transport and planting of ivy's on the roof grids of the steel canopies. |  |  |  |  |
| 344 | Canopy 1 | pcs |  | 1.00 |  |
| 345 | Canopy 2 | pcs |  | 1.00 |  |
| 346 | Canopy 3 | pcs |  | 1.00 |  |
| 347 | Canopy 4 | pcs |  | 1.00 |  |
| Total 4 | Total of CANOPIES |  |  |  |  |
| Total | Total of phase ARCHITECTURE | Total = Total of preparation works+ Total of green islands + Total of urban equipment + Total of Canopies |  |  |  |

- Phase – ELECTRICAL WORKS

| PRICE No | TITLE | UNIT | UNIT PRICE | ESTIMATED QUANTITIES | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| (a) | (b) | (c) | (d) | (e) | (f = d * e) |
| 400-500 | ELECTRICAL WORKS |  |  |  |  |
|  | Procurement and installation of parks lighting luminaires identical to or equivalent to the ones specified . |  |  |  |  |
| 401 | Schrack, RUBINO LED 53W, 6500Lm, 4000K, IP66, 100 000h.They can be mounted on brackets from F42 to F60. | Pieces |  | 3.00 |  |
|  | Procurement and installation of a reflector identical to or equivalent to the one specified: |  |  |  |  |
| 402 | Schrack, CAPRI LED Basic 10W, 1200Lm, 4000K, IP65, IK07, 50 000h, symmetric | Pieces |  | 10.00 |  |
| 403 | Schrack, CAPRI LED Basic 100W, 12000Lm, 4000K, IP65, IK07, 50 000h | Pieces |  | 4.00 |  |
|  | Procurement and installation of a walkover floor light identical to or equivalent to the one specified: |  |  |  |  |
| 404 | Schrack, SOLASTO GU10 6W, 258Lm, 230V, 4000K, IP67, round, stainless steel | Pieces |  | 32.00 |  |
| 405 | Steel pole 8m for electric lighting, galvanized and painted with silver paint, including a distribution plate inside the pole with three 10A automatic circuit breakers, terminals for cable input and output, and 3x NYY-J-3x2.5mm2 conductors from the plate to the luminaire with a length of 7m, in 3 segments. At a height of 5m on the pole, a T-profile should be installed for mounting 2 reflectors. | Pieces |  | 3.00 |  |
| 406 | Steel pole 8m for electric lighting, galvanized and painted with silver paint, including a distribution plate inside the pole with three 10A automatic circuit breakers, terminals for cable input and output, and 3x NYY-J-3x2.5mm2 conductors from the plate to the luminaire with a length of 7m, in 3 segments. At a height of 5m on the pole, a T-profile should be installed for mounting two reflectors. | Pieces |  | 2.00 |  |
| 407 | Installation of a bracket on the pole with a length of 1.0m | Pieces |  | 3.00 |  |
| 408 | Installation of a T-profile on the pole with a length of 1.5m for attaching reflectors | Pieces |  | 2.00 |  |
| 409 | Installation of a T-profile on the pole with a length of 1.0m for attaching reflectors at a height of 5.0m on the pole. | Pieces |  | 5.00 |  |
|  | Procurement and laying of cable type |  |  |  |  |
| 410 | - NYY-J-5x4 mm2 - in an underground trench from RO to PS for irrigation | m1 |  | 250 |  |
| 411 | - NAYY-J-4x16 mm2 - in an underground trench from RO to parkslight | m1 |  | 140 |  |
| 412 | - NYY-2x1.5 mm2 - for a photocell | m1 |  | 15 |  |
| 413 | Procurement and installation of a photocell | piece |  | 1 |  |
| 414 | Excavation of soil of category III for the foundations of lamp posts (poles) with foundation dimensions. - 1.0x1.0x1.0 m = 1.0m3 x 5 | m3 |  | 5 |  |
| 415 | Excavation of soil of category III for laying cables and grounding tape, with backfilling, compaction, and disposal of excess soil, with specified dimensions. - 0.4x0,8x250m | m3 |  | 80 |  |
| 416 | Spreading a layer of sand with a thickness of 10cm below and above the laid cables: - 0.4x0,2x250м | m3 |  | 20 |  |
| 417 | Supply and installation of a standard concrete foundation with concrete grade MB35 for installing lamp posts, including an embedded anchor plate and a pipe with a diameter of F76mm for cable entry and exit, with specified dimensions - 1.0x1.0x1.0 m (8m pole) | piece |  | 5 |  |
| 418 | Trenching the grounding tape FeZn 25x4mm with hot bitumen for 30cm until it exits the ground and reaches the connection point on the pole, as well as at the cross-joint of the tape in the ground | kg |  | 1.00 |  |
| 419 | Installation of protective conduits (shields) and warning tape above the cables in the trench | m |  | 250.00 |  |
| 420 | Connection of cable ends to distribution panels in lighting poles with cable glands suitable for operation | pieces |  | 20,00 |  |
| 421 | Grounding tape FeZn 25x4mm laid in a pre-excavated trench, with supply lines for connecting to the poles, metal fencing, and metal canopies | m1 |  | 280 |  |
| 422 | To form a distribution cabinet for parks lighting RO according to a single-line diagram. The panel should be made of double-decapitated sheet metal with a thickness of 2mm and painted for installation on a concrete foundation. The door should lock with a key. (PO should comply with the following standards: EN 60439-3 class 3, EN 60695, EN 60529 IP 65, EN 62262 IK 07) The following electrical equipment from a renowned manufacturer should be installed in the cabine - 1 piece of automatic circuit breaker AS63/R40A - 1 piece of circuit breakers CNM - 2 piece of knife switches 10-53-y - 1 piece of Fuse 26 v Automatic circuit breakers: - 4 piece B 20A, 3P - 1 piece B 10A, 1P - 3 piece B 16A, 1Preserve - 1 piece B 10A, 1Preserve - busbars for divorce: 5бр.(L1,L2,L3,PE,N) - other small material (various terminals, inscription plates, etc.) The wardrobe should be built according to a single-pole scheme with a 20% reserve | pieces |  | 1.00 |  |
| 423 | Procurement and issuance of certification documentation required for technical acceptance of the facility | Lump sum |  | 1,00 |  |
| Total | Total of phase ELECTRICAL WORKS |  |  |  |  |

- Phase – IRRIGATION AND SEWAGE

| PRICE No | TITLE | UNIT | UNIT PRICE | ESTIMATED QUANTITIES | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| (a) | (b) | (c) | (d) | (e) | (f = d * e) |
| 500-600 | I. SANITARY COLD WATER |  |  |  |  |
| 501 | Excavation of earth category III in a narrow volume with the provision of the trench for laying pipes 130.00 х 0.80 х 0.80 | m3 |  | 83.20 |  |
| 502 | Planning the bottom of the trench with an accuracy of +2 cm and spreading a layer of fine sand with a thickness of 10 cm: 130.00 х 0.80 х 0.10 | m3 |  | 10.40 |  |
| 503 | Backfilling the trench with excavated material after installation, conducting tests on the installation, and compacting in layers of 20-30 cm with compactors weighing 20-30 kg: 83,20 х 0.85 = | m3 |  | 70.72 |  |
| 504 | Transportation of excess material by tipper trucks up to 10 km distance, including manual loading: 83,20 х 0.15 = | m3 |  | 12.48 |  |
| 505 | Excavation of soil category III for a water meter shaft with provision of the pit: 1.30 х 1.60 х 1.30 | m3 |  | 2.70 |  |
| 506 | Concrete casting of a water meter shaft with MB20 concrete, with a payment layer up to 20 cm thick and reinforcement of F8. The internal walls plastered with cement mortar 1:2 to a smooth black finish and installation of rungs F16. Calculation for a fully concrete-cast water meter shaft. | pieces |  | 1,00 |  |
| 507 | Procurement, transport, and installation of a cast iron lid for the water meter shaft. | pieces |  | 1,00 |  |
| 508 | Procurement, transport, and installation of a water meter housed in a water meter shaft with a diameter of F 3/4 | pieces |  | 1,00 |  |
| 509 | Procurement, transport, and installation of connection fittings, bends, bends, reducers, etc., For one water meter. Calculation for a fully installed water meter shaft. | pieces |  | 1,00 |  |
| 510 | Manufacture of a connection to the existing water supply network | pieces |  | 1,00 |  |
|  | Procurement, transport, and installation of polypropylene pipes for cold water, complete with fittings and joint materials. Calculation for 1 meter of installed pipe.. |  |  |  |  |
| 511 | DN 20 | m1 |  | 8.00 |  |
| 512 | DN 25 | m1 |  | 94.00 |  |
| 513 | DN 32 | m1 |  | 40.00 |  |
| 514 | Procurement, transport, and installation of regular gate valves F 3/4“ | pieces |  | 3,00 |  |
| Total 1 | Total of SANITARY COLD WATER |  |  |  |  |
|  | WATER SUPPLY |  |  |  |  |
| 601 | Excavation of grade III soil in a narrow area with provision for trenching: 220.00 x 0.80 x 0.80 = | m3 |  | 140.80 |  |
| 602 | Planning the trench bottom with an accuracy of +2 cm and spreading fine sand with a thickness of 10 cm: 220.00 x 0.80 x 0.10 = | m3 |  | 17.60 |  |
| 603 | Backfilling the trench with excavated material after installation and testing of the installation and compacting in layers of 20-30 cm with compactors weighing 20-30 kg: 140.80 x 0.85 | m3 |  | 119.68 |  |
| 604 | Transport of excess material by dump trucks within a distance of up to 10 km, including manual loading: 140.80 x 0.15 | m3 |  | 21.12 |  |
| 605 | Excavation of grade III soil for a pump shaft to increase pressure with provision for the pit: 1.30 x 1.60 x 1.30 | m3 |  | 6.50 |  |
| 606 | Concrete pouring for the pump water supply shaft with concrete grade MB 20, including a layer with a thickness of up to 20 cm and reinforcement with diameter F8 mm. Internal walls to be plastered with a cement mortar mix of 1:2 to a smooth, black gloss finish, and installation of ladder rungs with a diameter of F16. Calculation for a fully concreted water supply shaft. | piece |  | 1 |  |
| 607 | Supply, transport, and installation of a cast iron lid for the water supply shaft. | piece |  | 1 |  |
| 608 | Procurement, transportation, and installation of a booster station to increase water pressure for the water system with two pumps (one operational and one reserve) with flexible connection and self-control, featuring the following specifications: for Q= 3.0 l/s I H=5m N =2 x 1,5 kw. Calculation for a fully installed booster station. | piece |  | 1 |  |
| 609 | Supply, transportation, and installation of ordinary valves in the water supply shaft F 2 1/2 | piece |  | 3 |  |
| 610 | Supply, transportation, and installation of a suction basket on the river inlet F 2 1/2“ | piece |  | 1 |  |
|  | Procurement, transportation, and installation of polypropylene pipes for cold water, complete with fitting components and joint materials. Calculation for 1 meter of installed pipe.. |  |  |  |  |
| 611 | DN 32 | m1 |  | 66.00 |  |
| 612 | DN 40 | m1 |  | 24.00 |  |
| 613 | DN 50 | m1 |  | 48.00 |  |
| 614 | DN 63 | m1 |  | 95.00 |  |
| 615 | Excavation of soil, III category, for gardening hydrant shafts with excavation pit preparation 0,90 х 0,90 х 0.80 х 11 = | m3 |  | 7.20 |  |
| 616 | Concrete work for hydrant shafts with MB 20, including a concrete thickness of up to 20 cm and reinforcement of F8. Internal walls to be plastered with cement mortar 1:2 to a black shine, and installation of F16 rungs. Calculation for a fully concrete-installed gardening hydrant shaft | piece |  | 11 |  |
| 617 | “Procurement, transportation, and installation of a gardening hydrant placed in a water supply mainhoole F 1 | piece |  | 11 |  |
| 618 | Procurement, transportation, and installation of a cast iron lid for the gardening hydrant shaft | piece |  | 11 |  |
| 619 | Hydraulic testing of the water supply network | m1 |  | 233.00 |  |
| Total 2 | Total of WATERSUPPLY |  |  |  |  |
| Total | TOTAL of IRRIGATION and SEWAGE | Total = Total of sanitary cold water + Total of watersupply |  |  |  |

| 1. Total of architecture (in EUR) |  |
| --- | --- |
| 2.Total of electrical works (in EUR) |  |
| 3.Total of irrigation and sewage (in EUR) |  |
| Total (1+2+3) without VAT |  |
| VAT 18% of Total |  |
| GRAND TOTAL with VAT 18% |  |

Note: the numbering of prices under (a) and titles under (b) correspond to the numberingin the price schedule

VOLUME 4.3.3 — PRICE SCHEDULE

| PRICE No 001 Title: PREPARATION COSTS Planning and clearing the terrain |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for planning and marking of terrain proposed for construction works and it includes: - Planning and clearing the terrain with marking and staking of green islands, flowerbeds, etc., according to elevation data from the location urbanistic part with a surveyor and geometric parameters. The prize is in m2 |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No 002 Title: PREPARATION COSTS Mechanical excavation of soil |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for the excavation of the soil for the need of foundation strips and it includes: - Mechanical excavation of soil for foundation strips, in a narrow scale of II-III category. The price must be with included loading and transportation surplus soil from excavations to a landfill in distance of 20 km. The prize is per m3 . |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No 003-004 Title: PREPARATION COSTS Concreting of retaining wall |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is supply of concrete AB for constructing of retaining wall under the stands of islands 1 and 2 and it includes: - Concreting of AB retaining wall under the stand of island 1 and island 2 with dimensions 20/50 cm with foundation strip 40/40 cm with MB30 in adequate thickness . The prize is per m3 . **remark – the price per m3 is equal for both stands |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No 005-006 Title: PREPARATION COSTS Reinforcement of the strips and retaining wall |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for purchasing of reinforcement for the foundation of the strips and walls and it includes: - Procurement, transportation, fabrication and installation of reinforcement in foundation strips and retaining walls at basketball court bleachers. It must be calculated in price per kgr **remark – the price per kgr is equal for both stands |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |
| PRICE No 007 Title: PREPARATORY WORKS – FENCING TO NEIGHBOR (Mechanical excavation of soil) |  |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for excavation of soil and it includes: - Mechanical excavation of soil for foundation strips, in a narrow scale of II-III category. The price must be with included loading and transportation surplus soil from excavations to a landfill in distance of 20 km. The prize is per m3 . |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No 008 Title: PREPARATORY WORKS – FENCING TO NEIGHBOR Concreting of foundation strips for fencing |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is supply of concrete AB for constructing of foundation strips for a fence to the neighbour and it includes: - Pouring reinforced concrete (AB) foundation strips for a fence to the neighbor with dimensions of 60/60 cm and an RC wall of 20/170 cm using MB 30 concrete as required. The price must be presented per m3 |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No 009 Title: PREPARATORY WORKS – FENCING TO NEIGHBOR Filling, compaction and tamping |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for filling and tamping and it includes: - Filling and compaction and tamping with varying thickness, in layers of 20-30 cm each, where the minimum compaction modulus of the final cushion layer should be Mv > 40 MPa, and the minimum compaction γ = 21.00 kN/m³, at the foundation strip. The price must be presented in EUR per m3 |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No 010 Title: PREPARATORY WORKS – FENCING TO NEIGHBOR Reinforcing the foundation strips |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for filling and tamping and it includes: - Procurement, transportation, fabrication, and installation of reinforcement in AB foundation strips and AB retaining wall adjoining the neighbour. The price must be presented in EUR per kgr for the proposed dimensions according the schemes and drawings |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No 101 Title: GREEN ISLANDS - SPACE BETWEEN GREEN ISLANDS Excavation of soil |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for excavation of soil and it includes: - Mechanical excavation of soil in a large volume, categories 3 and 4, 90% mechanical and 10% manual, for leveling the space between islands, including the removal of excavated soil to a landfill within 20 km. The price must be presented per m3 for the proposed dimensions according the schemes and drawings |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No.102 Title: GREEN ISLANDS - SPACE BETWEEN GREEN ISLANDS Filling, compaction and tamping |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for filling, compaction and tamping and it includes: - Filling and compaction and tamping with varying thickness, in layers of 20-30 cm each, where the minimum compaction modulus of the final cushion layer should be Mv > 40 MPa, and the minimum compaction γ = 21.00 kN/m³, at the space between the islands. The price must be presented per m3 for the proposed dimensions according the schemes and drawings |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No.103 Title: GREEN ISLANDS - SPACE BETWEEN GREEN ISLANDS Montage of geotextile |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for montage of geotextile and it includes: - Procurement, transportation, and montage of geotextile (felt) placed on a gravel substrate to form pathways for movement.The price must be presented per m2 for the proposed dimensions according the schemes and drawings |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |
| PRICE No.104 Title: GREEN ISLANDS - SPACE BETWEEN GREEN ISLANDS Filling and levelling of humus |  |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for filling of humus and it includes: - Filling and leveling a layer of humus with d=10 cm, in the space between the islands under the pavel elements.The price must be presented per m2 for the proposed dimensions according the schemes and drawings |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No.105 Title: GREEN ISLANDS - SPACE BETWEEN GREEN ISLANDS Montage of paver elements |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for montage of paver elements and it includes: - Procurement, transportation and installation of square paver elements 60x40 cm, d=10 cm on a substrate of sand and humus for a path. The sand pad is part of this position.The price must be presented per m2 for the proposed dimensions according the schemes and drawings |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No.106 Title: GREEN ISLANDS 1,2,3,4,5,6 Preparing of islands terrain |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for preparing of island terrain and it includes: - Earth filling, mechanical, with leveling and compaction of the terrain to the appropriate density for the entire designated area to create green islands according to the provided details. The price must be presented per m2 for the proposed dimensions according the schemes and drawings |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No.107 Title: GREEN ISLANDS 1,2,3,4,5,6 Placing agrofolia under the humus |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for placing the agrofolia and it includes: - Procurement, transport and installation of agrofolia under the incorporated humus in all green islands.. The price must be presented per m2 for the proposed dimensions according the schemes and proposed bill of quantities |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No.108-118 Title: GREEN ISLANDS 1,2,3,4,5,6 Greening |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for Greening of the areas within the green islands, which include the following positions: - topsoil filling + including soil delivery and spreading. - greening - grass seeding (leveling the surface, forming a lawn with grass seed) The price must be presented per m2 for the proposed dimensions according the schemes and proposed bill of quantities **remark – the price per m2 is equal for all positions |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No.119-121 Title: GREEN ISLANDS 1,2,3,4,5,6 Placing agrofolia under the humus |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for constructing of stone slabs and it includes: - transportation, and installation of stone slabs with dimensions of 200/70/12 cm on a concrete base with a thickness of 8 cm to form bleachers at the basketball court . The price must be presented per m3 for the proposed dimensions according the schemes and proposed bill of quantities |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No.122-137 Title: GREEN ISLANDS 1,2,3,4,5,6 Planting |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for planting it includes: - Procurement, transport and planting of a dendrological type of plant defined by the designer. The price must be presented per piece of each item Pinus nigra (h=1.80m2.00m), Cotoneaster dammeri – bella (Ф=40-50сm), Acer palmatum atropurpureum, h=1.50-1.60m, Cotoneaster dammeri – bella (Ф=40-50сm) Salix babylonica h=3.0m, Alnus spp.h=2.00m, Prunus laurocerasus otto luyken (F=50-60см), (Platanus SPP - Platanus × acerifolia), h=4.0м, (HEDERA HELIX) (h=50-60cm), (Cotoneaster dammeri – bella) (Ф=40-50сm), (picea pungens glauca) (h=1.5-1.6m), (Fagus atropurpurea) (h=1.5-1.6m), (Fagus moesiaca)(h=1.5-1.6m), (Prunus laurocerasus otto luyken) (Ф=50-60см), (Vinca minor) (расад Р9), (Salix babylonica) (h=3.0m) |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No.138-143 Title: GREEN ISLANDS 1,2,3,4,5,6 Construction of concrete curbstones |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for montage of concrete curbstones and it includes: - Procurement, transportation, and installation of concrete curbstones 7/20/50 cm made of MB 40 placed on a concrete base of MB 20 (park curb) (The concrete base is part of this position). The price must be presented per m1 per each island for the proposed dimensions according the schemes and proposed bill of quantities |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No.144-146 Title: GREEN ISLANDS 1,2,3,4,5,6 Flowering of the islands |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for flowering of the all 6 islands and it includes: - Procurement, transportation, and planting of a flowerbed with seasonal flowering plants at a rate of 25 flowers per square meter The price must be presented per pieces per each island |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No.201 Title: URBAN EQUIPMENT Waste bins |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for montage of waste bins it includes: - Procurement, transportation, and installation of waste bins made from wooden planks arranged in a circle, held by two rings, made of metal flat bars 40/4 mm (bottom and top) on a metal box-profile frame. The planks should be fixed to the metal flat bars. The price must be presented per piece of each item |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No.202 Title: URBAN EQUIPMENT Taps |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for montage of taps and it includes: - Procurement, transport and installation of taps for drinking water, according to the investor's choice. The taps should have a built-in valve for turning on and off the water. The planks should be fixed to the metal flat bars. The price must be presented per piece of each item |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No.203 Title: URBAN EQUIPMENT Benches |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for montage of benches and it includes: - Procurement, transportation, and installation of benches (the benches to be anchored on two reinforced concrete foundation blocks with dimensions of 20x45 cm and a depth of 25 cm). The seating benches to be made from stamped metal construction of box profiles.. The price must be presented per piece of each item |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |
| PRICE No.301-328 Title: CANOPIES Montage of steel canopies |  |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for montage of canopies: - Preparing, transport and montage of steel canopies miniaturized and oil-painted, consisting of circular profiles F100/4mm and covered wiith solar panels 100/200 mm placed on steel profiles 120.100.5 mm and substructure made of steel profiles 50.50.4 mm according to all standards for this type type of work. It is possible to eventually add couplings depending on the weight of the vertical couplings.. It includes: - Tiles 200.200.10 mm / Pillars101,6 5 мм / 11,9 kg/m / Beams 120.100.5 мм / 22,4 kg/m / substructure 50.50.4 мм / 5,51 kg/m / couplings – horizontal / sand + lime 10% The price must be presented per kgr per each item of each 4 canopies |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |
| PRICE No.329 Title: CANOPIES Montage of gutters |  |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for montage of gutters and it includes : - Preparation, transport and montage of horizontal hanging gutters in canopies with R.W=75 cm from plasticized, galvanized steel sheet with d=0.55 mm. The compositions made according to technical regulations, welded and with simultaneous production of fall %. Complete with binding material and necessary brackets (hooks and claws protected against corrosion). The price must be presented per m2 per each item |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No.330 Title: CANOPIES Montage of hanging gutters |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for montage of hanging gutters: - Preparation, transport and montage of horizontal hanging gutters in canopies with R.W=75 cm from plasticized, galvanized steel sheet with d=0.55 mm. The compositions made according to technical regulations, welded and with simultaneous production of fall %. Complete with binding material and necessary brackets (hooks and claws protected against corrosion) The price must be presented per m2 per each item |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |
| PRICE No.329 Title: CANOPIES Montage of gutters |  |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for montage of gutters and it includes : - Preparation, transport and montage of horizontal hanging gutters in canopies with R.W=75 cm from plasticized, galvanized steel sheet with d=0.55 mm. The compositions made according to technical regulations, welded and with simultaneous production of fall %. Complete with binding material and necessary brackets (hooks and claws protected against corrosion). The price must be presented per m2 per each item |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No.330 Title: CANOPIES Montage of hanging gutters |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for montage of hanging gutters: - Preparation, transport and montage of horizontal hanging gutters in canopies with R.W=75 cm from plasticized, galvanized steel sheet with d=0.55 mm. The compositions made according to technical regulations, welded and with simultaneous production of fall %. Complete with binding material and necessary brackets (hooks and claws protected against corrosion) The price must be presented per m2 per each item |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |
| PRICE No.331 Title: CANOPIES Excavation of soil |  |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for excavation of soil and it includes : - Mechanical excavation of soil for foundation strips, in a narrow scale, of II-III category with included loading and transportation surplus soil from excavations to a landfill in distance of 20 km. The price must be presented per m3 |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No.330 Title: CANOPIES Montage of hanging gutters |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for montage of hanging gutters: - Preparation, transport and montage of horizontal hanging gutters in canopies with R.W=75 cm from plasticized, galvanized steel sheet with d=0.55 mm. The compositions made according to technical regulations, welded and with simultaneous production of fall %. Complete with binding material and necessary brackets (hooks and claws protected against corrosion) The price must be presented per m2 per each item |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |
| PRICE No.332 Title: CANOPIES Excavation of soil |  |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for excavation of soil and it includes : - Mechanical excavation of soil for foundation strips, in a narrow scale, of II-III category with included loading and transportation surplus soil from excavations to a landfill in distance of 20 km. The price must be presented per m3 |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No 332-335 Title: CANOPIES Concreting of foundations for canopy |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for concreting of foundation for canopies and it includes: - Concreting of AB single foundations for steel canopy with solar panels of dimensions 80/80/50 cm with pillars 30/30/40 cm with MB 30 in required thickness.. The price must be presented per m3 per each canopy for the proposed dimensions according the schemes and drawings |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No 336-339 Title: CANOPIES Reinforcing the foundations of canopy |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for reinforcement of foundation of canopies and it includes: - Procurement, transportation, fabrication and installation of reinforcement inABfoundation singles for canopy.. The price must be presented per kgr for the proposed dimensions according the schemes and drawings for all canopies |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No 340-343 Title: CANOPIES Filling and compacting a buffer |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for creating a buffer and it includes: - Filling and compacting a buffer with a thickness of d=40cm, in layers of 20cm each, where the minimum compaction modulus of the final buffer layer should be Mv>40MPa and the minimum compaction ϒ=21.00kN/m³, under the foundations of steel canopies . The price must be presented per m3 for the proposed dimensions according the schemes and drawings |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No 344-347 Title: CANOPIES Planting a roofs of canopies |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for planting a roofs of canopies and it includes: - Procurement, transport and planting of ivy's on the roof grids of the steel canopies. . The price must be presented per pieces for all canopies |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No 401 Title: ELECTRICAL WORKS Parks lightning |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for installation of parks lightning and it includes: - Procurement and installation of parks lighting luminaires identical to or equivalent to the ones specified . Schrack, RUBINO LED 53W, 6500Lm, 4000K, IP66, 100 000h. They can be mounted on brackets from F42 to F60. The price must be presented per pieces. |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No 402-404 Title: ELECTRICAL WORKS Parks lightning |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for installation of reflectors and it includes: - Procurement and installation of a reflector identical to or equivalent to the one specified: - Schrack, RUBINO LED 53W, 6500Lm, 4000K, IP66, 100 000h. Schrack, CAPRI LED Basic 10W, 1200Lm, 4000K, IP65, IK07, 50 000h, symmetric. - Schrack, CAPRI LED Basic 100W, 12000Lm, 4000K, IP65, IK07, 50 000h - Schrack, CAPRI LED Basic 100W, 12000Lm, 4000K, IP65, IK07, 50 000h The price must be presented per pieces. |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No 405-409 Title: ELECTRICAL WORKS Poles for lightning |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for montage and installation of electrical poles and it includes: - Steel pole 8m for electric lighting, galvanized and painted with silver paint, including a distribution plate inside the pole with three 10A automatic circuit breakers, terminals for cable input and output, and 3x NYY-J-3x2.5mm2 conductors from the plate to the luminaire with a length of 7m, in 3 segments. At a height of 5m on the pole, a T-profile should be installed for mounting 2 reflectors. - Steel pole 8m for electric lighting, galvanized and painted with silver paint, including a distribution plate inside the pole with three 10A automatic circuit breakers, terminals for cable input and output, and 3x NYY-J-3x2.5mm2 conductors from the plate to the luminaire with a length of 7m, in 3 segments. At a height of 5m on the pole, a T-profile should be installed for mounting two reflectors. - Installation of a bracket on the pole with a length of 1.0m - Installation of a T-profile on the pole with a length of 1.5m for attaching reflectors - Installation of a T-profile on the pole with a length of 1.0m for attaching reflectors at a height of 5.0m on the pole. The price must be presented per pieces. |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No 410-412 Title: ELECTRICAL WORKS Poles for lightning |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for montage and installation of electrical poles and it includes: - NYY-J-5x4 mm2 - in an underground trench from RO to PS for irrigation - NAYY-J-4x16 mm2 - in an underground trench from RO to parkslight - NYY-2x1.5 mm2 - for a photocell The price must be presented per m1 . |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No 413 Title: ELECTRICAL WORKS Photocell |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for installation of a photocell and it includes: Procurement and installation of a photocell The price must be presented per pieces. |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No 414 Title: ELECTRICAL WORKS Excavations |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for excavationl and it includes: Excavation of soil of category III for the foundations of lamp posts (poles) with foundation dimensions. - 1.0x1.0x1.0 m = 1.0m3 x 5. The price must be presented per m3 . |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No 415 Title: ELECTRICAL WORKS Excavations |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for excavationl and it includes: Excavation of soil of category III for laying cables and grounding tape, with backfilling, compaction, and disposal of excess soil, with specified dimensions. - 0.4x0,8x250m. The price must be presented per m3 . |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No 416 Title: ELECTRICAL WORKS Layer of sand |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for layer of sand and it includes: Spreading a layer of sand with a thickness of 10cm below and above the laid cables: - 0.4x0,2x250м. The price must be presented per m3 . |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No 417 Title: ELECTRICAL WORKS Foundation for light posts |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for mounting of foundation for light posts and it includes: - Supply and installation of a standard concrete foundation with concrete grade MB35 for installing lamp posts, including an embedded anchor plate and a pipe with a diameter of F76mm for cable entry and exit, with specified dimensions - 1.0x1.0x1.0 m (8m pole)- 0.4x0,2x250м. The price must be presented per pieces . |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No 418 Title: ELECTRICAL WORKS Grounding |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for grounding and it includes: - Trenching the grounding tape FeZn 25x4mm with hot bitumen for 30cm until it exits the ground and reaches the connection point on the pole, as well as at the cross-joint of the tape in the ground. The price must be presented per kgr . |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No 419 Title: ELECTRICAL WORKS Warning tape |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for laying a warning tape and it includes: - Installation of protective conduits (shields) and warning tape above the cables in the trench The price must be presented per m1 . |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No 420 Title: ELECTRICAL WORKS Connections |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for connecting and it includes: - Connection of cable ends to distribution panels in lighting poles with cable glands suitable for operation The price must be presented per pieces . |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No 421 Title: ELECTRICAL WORKS Connecting of grounding |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for connecting of grounding and it includes: - Grounding tape FeZn 25x4mm laid in a pre-excavated trench, with supply lines for connecting to the poles, metal fencing, and metal canopies The price must be presented per m1 . |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No 422 Title: ELECTRICAL WORKS Command switch box |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for installation of command switch box and it includes: To form a distribution cabinet for parks lighting RO according to a single-line diagram. The panel should be made of double-decapitated sheet metal with a thickness of 2mm and painted for installation on a concrete foundation. The door should lock with a key. (PO should comply with the following standards: EN 60439-3 class 3, EN 60695, EN 60529 IP 65, EN 62262 IK 07) The following electrical equipment from a renowned manufacturer should be installed in the cabine - 1 piece of automatic circuit breaker AS63/R40A - 1 piece of circuit breakers CNM - 2 piece of knife switches 10-53-y - 1 piece of Fuse 26 v Automatic circuit breakers: - 4 piece B 20A, 3P - 1 piece B 10A, 1P - 3 piece B 16A, 1Preserve - 1 piece B 10A, 1Preserve - busbars for divorce: 5бр.(L1,L2,L3,PE,N) - other small material (various terminals, inscription plates, etc.) - The wardrobe should be built according to a single-pole scheme with a 20% reserve The price must be presented per pieces. |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No 423 Title: ELECTRICAL WORKS Technical documentation |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for connecting of grounding and it includes: - Procurement and issuance of certification documentation required for technical acceptance of the facility. The price must be presented per m1 . |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No 501 Title: IRRIGATION AND SEWAGE SANITARY COLD WATER Excavations |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for excavation and it includes: - Excavation of earth category III in a narrow volume with the provision of the trench for laying pipes 130.00 х 0.80 х 0.80. The price must be presented per m3 . |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No 502 Title: IRRIGATION AND SEWAGE SANITARY COLD WATER Laying sand |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for laying a sand and it includes: - Planning the bottom of the trench with an accuracy of +2 cm and spreading a layer of fine sand with a thickness of 10 cm: 130.00 х 0.80 х 0.10. The price must be presented per m3 . |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No 503 Title: IRRIGATION AND SEWAGE SANITARY COLD WATER Compacting a layer |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for compacting a layer of sand and it includes: - Backfilling the trench with excavated material after installation, conducting tests on the installation, and compacting in layers of 20-30 cm with compactors weighing 20-30 kg: 83,20 х 0.85 =. The price must be presented per m3 . |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No 504 Title: IRRIGATION AND SEWAGE SANITARY COLD WATER Transportation of materials |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for transportation of material and it includes: - Transportation of excess material by tipper trucks up to 10 km distance, including manual loading: 83,20 х 0.15 The price must be presented per m3 . |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No 505 Title: IRRIGATION AND SEWAGE SANITARY COLD WATER Excavations |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for excavation and it includes: - Excavation of soil category III for a water meter shaft with provision of the pit: 1.30 х 1.60 х 1.30 The price must be presented per m3 . |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No 506 Title: IRRIGATION AND SEWAGE SANITARY COLD WATER Water meter shaft |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for construction of water meter shaft and it includes: - Concrete casting of a water meter shaft with MB20 concrete, with a payment layer up to 20 cm thick and reinforcement of F8. The internal walls plastered with cement mortar 1:2 to a smooth black finish and installation of rungs F16. Calculation for a fully concrete-cast water meter shaft. The price must be presented per pieces . |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No 507 Title: IRRIGATION AND SEWAGE SANITARY COLD WATER Water meter shaft |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for montage of water meter shaft and it includes: - Procurement, transport, and installation of a cast iron lid for the water meter shaft. The price must be presented per pieces. |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No 508 Title: IRRIGATION AND SEWAGE SANITARY COLD WATER Water meter shaft |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for montage of water meter shaft and it includes: - Procurement, transport, and installation of a water meter housed in a water meter shaft with a diameter of F 3/4 . The price must be presented per pieces. |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No 509 Title: IRRIGATION AND SEWAGE SANITARY COLD WATER Water meter shaft |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for installation of elements for water meter shaft and it includes: - Procurement, transport, and installation of connection fittings, bends, bends, reducers, etc., For one water meter. Calculation for a fully installed water meter shaft. The price must be presented per pieces. |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No 510 Title: IRRIGATION AND SEWAGE SANITARY COLD WATER Water meter shaft |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for connecting of water meter shaft and it includes: - Manufacture of a connection to the existing water supply network The price must be presented per pieces. |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No 511-513 Title: IRRIGATION AND SEWAGE SANITARY COLD WATER Fitting the pipes |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for installation and fitting the pipes and it includes: - Procurement, transport, and installation of polypropylene pipes for cold water, complete with fittings and joint materials. Calculation for 1 meter of installed pipe. Pipes DN-20 , DN-25 and DN-32 The price must be presented per m1 |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No 514 Title: IRRIGATION AND SEWAGE SANITARY COLD WATER Valves |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for installation and fitting the valves and it includes: - Procurement, transport, and installation of regular gate valves F 3/4“ The price must be presented per pieces. |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No 601 Title: IRRIGATION AND SEWAGE WATER SUPPLY Excavations |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for excavation of soil and it includes: - Excavation of grade III soil in a narrow area with provision for trenching: 220.00 x 0.80 x 0.80 The price must be presented per pieces. |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No 602 Title: IRRIGATION AND SEWAGE WATER SUPPLY Trench bottom |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for planning a trench bottom and it includes: - Planning the trench bottom with an accuracy of +2 cm and spreading fine sand with a thickness of 10 cm: 220.00 x 0.80 x 0.10 = The price must be presented per m3. |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No 603 Title: IRRIGATION AND SEWAGE WATER SUPPLY Filling and compacting bottom |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for filling and compacting and it includes: - Backfilling the trench with excavated material after installation and testing of the installation and compacting in layers of 20-30 cm with compactors weighing 20-30 kg: 140.80 x 0.85 = The price must be presented per m3. |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

v

| PRICE No 604 Title: IRRIGATION AND SEWAGE WATER SUPPLY Transport of excess material |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for transport of materials and it includes: - Transport of excess material by dump trucks within a distance of up to 20 km, including manual loading: 140.80 x 0.15 The price must be presented per m3. |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No 605 Title: IRRIGATION AND SEWAGE WATER SUPPLY Excavation of soil |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for filling and compacting and it includes: - Excavation of grade III soil for a pump shaft to increase pressure with provision for the pit: 1.30 x 1.60 x 1.30 The price must be presented per m3. |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No 606 Title: IRRIGATION AND SEWAGE WATER SUPPLY Concreting the shaft |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for filling and concreting the shaft and it includes: - Concrete pouring for the pump water supply shaft with concrete grade MB 20, including a layer with a thickness of up to 20 cm and reinforcement with diameter F8 mm. Internal walls to be plastered with a cement mortar mix of 1:2 to a smooth, black gloss finish, and installation of ladder rungs with a diameter of F16. Calculation for a fully concreted water supply shaft. - The price must be presented per pieces. |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No 607 Title: IRRIGATION AND SEWAGE WATER SUPPLY Water supply shaft |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for installation of water supply shaft and it includes: - Supply, transport, and installation of a cast iron lid for the water supply shaft.. The price must be presented per pieces. |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No 608 Title: IRRIGATION AND SEWAGE WATER SUPPLY Booster station |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for installation ofbooster station and it includes: - Procurement, transportation, and installation of a booster station to increase water pressure for the water system with two pumps (one operational and one reserve) with flexible connection and self-control, featuring the following specifications: for Q= 3.0 l/s I H=5m N =2 x 1,5 kw. Calculation for a fully installed booster station... The price must be presented per pieces. |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No 609 Title: IRRIGATION AND SEWAGE WATER SUPPLY Ordinary valve |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for installation of ordinary valve and it includes: - Supply, transportation, and installation of ordinary valves in the water supply shaft F 2 The price must be presented per pieces. |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No 610 Title: IRRIGATION AND SEWAGE WATER SUPPLY Suction basket |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for installation of suction basket and it includes: - Supply, transportation, and installation of a suction basket on the river inlet F 2 1/2“ - The price must be presented per pieces. |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No 611-614 Title: IRRIGATION AND SEWAGE WATER SUPPLY Fitting of pipes and joint materials |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for installation of pipes and joint materials and it includes: - Procurement, transportation, and installation of polypropylene pipes for cold water, complete with fitting components and joint materials. Calculation for 1 meter of installed pipe.. (DN-32, DN-40, DN-50 and DN-63) The price must be presented per m1 . |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No 615 Title: IRRIGATION AND SEWAGE WATER SUPPLY Excavations |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for excavations and it includes: - Excavation of soil, III category, for gardening hydrant shafts with excavation pit preparation 0,90 х 0,90 х 0.80 х 11 The price must be presented per m3 . |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No 616 Title: IRRIGATION AND SEWAGE WATER SUPPLY Hydrant shafts |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for concreting and it includes: - Concrete work for hydrant shafts with MB 20, including a concrete thickness of up to 20 cm and reinforcement of F8. Internal walls to be plastered with cement mortar 1:2 to a black shine, and installation of F16 rungs. Calculation for a fully concrete-installed gardening hydrant shaft - The price must be presented per pieces.. |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No 617 Title: IRRIGATION AND SEWAGE WATER SUPPLY Hydrant shafts |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for montage of hydrant shaft and it includes: -Procurement, transportation, and installation of a gardening hydrant placed in a water supply mainhoole F 1 from the drawings. The price must be presented per pieces.. |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No 618 Title: IRRIGATION AND SEWAGE WATER SUPPLY Hydrant shafts |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for montage of hydrant lid and it includes: - Procurement, transportation, and installation of a cast iron lid for the gardening hydrant shaft. The price must be presented per pieces.. |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

| PRICE No 619 Title: IRRIGATION AND SEWAGE WATER SUPPLY Testing of the network |  |
| --- | --- |
| Description of the implementation modalities of the works comprised within this price, as well as the measurement modalities of the quantities executed and to be paid by this price. This price is for testing the network and it includes: - Hydraulic testing of the water supply network The price must be presented per m1.. |  |
| PRICE No … — TITLE: …. <for example: PRICE No 001 —EXCAVATE TOPSOIL WITHIN ROADWAY> |  |
| Price in words: <for example: per square meter: ….. EUR> | Price in numbers: <for example: ….. EUR > |

.